# VolksSwitch Keyguard — V2 Target Specification

Format version: V2 (current)

# Overview

The V2 O&A format is the current format produced by the migration tool and consumed by keyguard.scad. Every column is always present and has an explicit value — there are no blanks, no undef, and no omitted trailing columns.

No version header is required. Format is auto-detected per vector: a row is V2 when column index 1 holds a quoted string (the shape name). A row is V1 when column index 1 is a number (the x coordinate). Each of the four vectors (screen_openings, case_openings, tablet_openings, case_additions) is detected independently, so a file can mix V1 and V2 vectors.

# Sections

| Section | Columns | Purpose |
| --- | --- | --- |
| screen_openings | 14 | Openings relative to the screen area |
| case_openings | 14 | Openings relative to the case opening |
| tablet_openings | 14 | Openings relative to the tablet body |
| case_additions | 8–9 | Solid shapes added on top of the case opening |

# V2 Row Format — screen_openings, case_openings, tablet_openings (14 columns)

[ID, shape, height, width, corner, x, y, cut/build, anchor, surface, length, thickness, [extra_slopes], [special]]
 [0]  [1]    [2]    [3]    [4]    [5] [6]    [7]      [8]    [9]     [10]    [11]           [12]            [13]

## Column Definitions

| # | Name | Type | V2 requirement |
| --- | --- | --- | --- |
| 0 | ID | number, "#", or quoted string | Always present |
| 1 | shape | quoted string | Always present |
| 2 | height | number | Always explicit |
| 3 | width | number | Always explicit (0 if unused by shape) |
| 4 | corner | number | Always explicit (0 if unused) |
| 5 | x | number | Always explicit |
| 6 | y | number | Always explicit |
| 7 | cut/build | number | Always 0 (= full depth) or explicit value; never blank/undef |
| 8 | anchor | "L" or "C" | Always uppercase; never blank |
| 9 | surface | "T" or "B" | Always uppercase; never blank |
| 10 | length | number | Always explicit (0 if unused) |
| 11 | thickness | number | Always explicit (0 if unused) |
| 12 | extra_slopes | sub-array | Always present; [] if unused |
| 13 | special | sub-array | Always present; [] if unused |

## Semantics of cut/build = 0

When cut/build (column 7) is 0, it means "use full depth" — the opening or shape extends all the way through the keyguard. It does not mean zero depth. The OpenSCAD dispatcher converts 0 to undef before passing it to cut_opening(), which then uses the full keyguard thickness as the depth.

## anchor values

| Value | Meaning |
| --- | --- |
| "L" | Lower-left anchor (default). The x/y coordinates specify the lower-left corner of the shape. |
| "C" | Centre anchor. The x/y coordinates specify the centre of the shape. |

## surface values

| Value | Meaning |
| --- | --- |
| "T" | Top surface (default). The shape is cut from or built on the top surface. |
| "B" | Bottom surface. The shape is cut from or built from the bottom surface upward. |

## Shape-specific column behaviour

| Shape | col [3] width | col [4] corner | col [7] cut/build | col [10] length | col [11] thickness |
| --- | --- | --- | --- | --- | --- |
| r, c, hd, oa1–4, r1–4 | Used | Used | Full depth if 0 | Unused (0) | Unused (0) |
| ridge, cridge, rridge, aridge1–4 | Unused (0) | Unused (0) | ridge_height | Used | Used |
| vridge, hridge | Unused (0) | Unused (0) | ridge_height | Used | Used |
| bump | Unused (0) | Unused (0) | Used | Unused (0) | Unused (0) |
| text | Unused (0) | Used | cut/build flag | Used | Used |
| svg | Used | Used | cut/build flag | Unused (0) | Unused (0) |

# V2 Row Format — case_additions (8–9 columns (variable))

Without cut/build:  [ID, shape, height, width, corner, x, y,      [trim]]
                     [0]  [1]    [2]    [3]    [4]   [5] [6]        [7]

With    cut/build:  [ID, shape, height, width, corner, x, y, cb,   [trim]]
                     [0]  [1]    [2]    [3]    [4]   [5] [6] [7]    [8]

cut/build (cb) is present when element [7] is a number. [trim] is always the last element and always a sub-array ([] if not needed).

Negative shapes (prefix "-", e.g. -r1, -tab2) create edge subtractions. The shapes "r", "-r", "c", and "-c" are not supported in V2 case_additions; use r1-4 instead.

| # | Name | V2 requirement |
| --- | --- | --- |
| 0 | ID | Always present |
| 1 | shape | Always present |
| 2 | height | Always explicit |
| 3 | width | Always explicit |
| 4 | corner | Always explicit (0 if unused) |
| 5 | x | Always explicit |
| 6 | y | Always explicit |
| 7 | cut/build | Optional number. Omit when full-height (default). Detected by checking whether element [7] is a number. |
| 8 | trim | Always the last element; [] if not needed |

# Example rows

// screen_openings -- standard rectangular opening, full depth, left-anchor, top surface
[1, "r", 45.0, 45.0, 2.0, 10.5, -22.3, 0, "L", "T", 0, 0, [], []],

// screen_openings -- ridge, ridge_height=1.5, length=45, base thickness=2, rightward
[2, "ridge", 3.0, 0, 0, 0, 0, 1.5, "L", "T", 45.0, 2.0, [], ["right"]],

// case_openings -- centre-anchored circle, full depth, bottom surface
[3, "c", 12.0, 12.0, 0, 0, 0, 0, "C", "B", 0, 0, [], []],

// case_additions -- r1 shape raised 5 mm thick, no trim
[4, "r1", 20.0, 15.0, 2.0, 5.0, 5.0, 5.0, []],

// case_additions -- full-height tab1 (no cut/build column)
[5, "tab1", 8.0, 6.0, 1.5, 0, 0, []],

// case_additions -- negative shape (edge subtraction), trimmed above at 10 mm
[6, "-r1", 10.0, 8.0, 0, 0, 0, [10, -999, -999, -999]],